1. Sensors (Input Stage)

- Camera:

  - Captures images.

  - Sends image frames to the Jetson for processing.

- LiDAR:

  - Captures 2D point clouds (environment scan).

  - Sends scan data to the Jetson.

2. Jetson (Processing Unit)

- Receives data from Camera and LiDAR.

- Processes the image and point cloud to identify the target coordinates.

- Sends the target coordinates as an Ethernet packet to Raspberry Pi 1 and Raspberry Pi 2 via the Ethernet Switch.

3. Ethernet Communication

- Ethernet Switch / Hub distributes the Ethernet packets (containing target coordinates).

- Both Raspberry Pi 1 and Raspberry Pi 2 receive the same target coordinates simultaneously.

4. Raspberry Pis (Local Control Units)

- Raspberry Pi 1:

  - Receives target coordinates.

  - Performs Inverse Kinematics (IK) to calculate required joint angles.

  - Sends CAN bus messages to motors of Leg 1 and Leg 2.

- Raspberry Pi 2:

  - Receives target coordinates.

  - Performs Inverse Kinematics (IK) to calculate required joint angles.

  - Sends CAN bus messages to motors of Leg 3 and Leg 4.

5. CAN Buses (Motor Communication Layer)

- Each Raspberry Pi has two CAN buses:

  - Pi 1 CAN Bus 0 → controls motors for Leg 1.

  - Pi 1 CAN Bus 1 → controls motors for Leg 2.

  - Pi 2 CAN Bus 0 → controls motors for Leg 3.

  - Pi 2 CAN Bus 1 → controls motors for Leg 4.

- Each CAN bus sends motor control commands (angles, torque) to the respective motors.

6. Leg Modules (Actuators)

- Each leg receives motor angle commands from the CAN bus.

- Each Leg contains:

  - Hip Motor 1

  - Hip Motor 2

  - Knee Motor

- These motors execute the calculated angles to move the leg accordingly.

Workflow Summary:

- Camera + LiDAR → provide environment data.

- Jetson → processes data, finds target coordinates.

- Ethernet → distributes target info to Raspberry Pis.

- Raspberry Pis → perform IK, compute motor angles.

- CAN Buses → transmit commands to motors.

- Legs → execute movement with Hip + Knee motors.